To:The Director,CHC Rest Houses (Pvt) Ltd,No. 327, Union Place,Colombo 02.

Demolishing and new brick wall for bar counter area updated rates.

| ITEM | DESCRIPTION | QTY | UNIT | RATE | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 1 | Carefully Demolishing 1'0" thick rubble wall including existing finishes and disposal of all debris from the site area as directed. | 210.00 | Cu/ft | 200.00 | 42,000.00 |
| 2 | Carefully Demolishing Existing slab and disposal of all debris from the site area as directed. | 140.00 | Sq/ft | 240.00 | 33,600.00 |
| 3 | Supply and apply grade 20 concrete including 6mm tor steel, 12mm tor steel, 6mm stirrups, formwork for 12" x 9" floor beam | 20.00 | L/ft | 2,400.00 | 48,000.00 |
| 4 | Brick work in 9" thick vertical walls in cement and sand 1:5 mortar in wall. (Rates revised to original because too low and can not finish the work to the rate) | 180.00 | Sq/ft | 750.00 | 135,000.00 |
| 5 | 16mm (5/8”) thick internal plaster 1:3 cement and sand finished smooth to the block walls, concrete walls, attached & isolated concrete columns. (Rates revised to original because too low and can not finish the work to the rate) | 370.00 | Sq/ft | 220.00 | 81,400.00 |
| 6 | Prepare plastered surfaces of brick walls, and apply two coats of approved interior quality, acrylic emulsion paint with two coat putty, two coat acrylic filler as per manufacturer's instructions, drawings and to the satisfaction of the Engineer and Architect. | 370.00 | Sq/ft | 235.00 | 86,950.00 |
|  | Grand Total |  |  |  | 426,950.00 |

@2026-02-16